JANANI J, 192512155.

# EXPERIMENT – 4

MATLAB SIMULATION OF SUPERHETERODYNE RECEIVER FOR AM COMMUNICATION SYSTEM WITH FREQUENCY MIXING AND FILTERING

## PROGRAM:

clc; clear; close all;

% Parameters fs = 100000;

t = 0:1/fs:0.02;

fm = 500; % Message frequency fc = 10000; % Carrier frequency

fLO = 14550; % Local Oscillator (IF = 4550 Hz)

% Message Signal m = cos(2*pi*fm*t);

% Carrier Signal

c = cos(2*pi*fc*t);

% AM Signal

am = (1 + m).*cos(2*pi*fc*t);

% Local Oscillator

LO = cos(2*pi*fLO*t);

% Frequency Mixing mixed = am .* LO;

% IF Filter

[b,a] = butter(4,5000/(fs/2)); IF = filter(b,a,mixed);

% Envelope Detection env = abs(hilbert(IF));

% Display figure; subplot(4,1,1) plot(t,m)

title('Message Signal') grid on

figure; subplot(5,1,2) plot(t,c) title('Carrier Signal') grid on subplot(5,1,3) plot(t,am)

title('AM Signal') grid on subplot(5,1,4) plot(t,IF)

title('Intermediate Frequency (IF) Signal') grid on

subplot(5,1,5) plot(t,env) title('Recovered Signal') grid on

## OUTPUT: