192512155.

# EXPERIMENT – 6

GENERATION AND ANALYSIS OF FREQUENCY MODULATED (FM) AND OF PHASE MODULATED (PM) SIGNALS USING MATLAB

PROGRAM:

clc; clear; close all; Am=1; Ac=1;

fm=5; fc=50; beta=5; kp=5; fs=1000;

t=0:1/fs:1;

m = Am*cos(2*pi*fm*t); % Message signal c = Ac*cos(2*pi*fc*t); % Carrier signal

fm_sig = Ac*cos(2*pi*fc*t + beta*sin(2*pi*fm*t)); % FM pm_sig = Ac*cos(2*pi*fc*t + kp*m); % PM

Df = beta*fm; % Frequency deviation fprintf('Frequency Deviation = %d Hz\n',Df); fprintf('FM Modulation Index = %d\n',beta); subplot(4,1,1), plot(t,m), title('Message Signal') subplot(4,1,2), plot(t,c), title('Carrier Signal') subplot(4,1,3), plot(t,fm_sig), title('FM Signal') subplot(4,1,4), plot(t,pm_sig), title('PM Signal')